# Full device configurations

### R-CAMP1

hostname R-CAMP1
no ip domain-lookup
ip domain-name campus.local
enable secret Cisco12345
service password-encryption
username netadmin privilege 15 secret Cisco12345
crypto key generate rsa modulus 1024
ip ssh version 2
!
track 1 interface GigabitEthernet0/1 line-protocol
!
interface GigabitEthernet0/0
 description Uplink to campus WAN
 ip address dhcp
 no shutdown
!
interface GigabitEthernet0/1
 description Trunk to DSW1
 no ip address
 no shutdown
!
interface GigabitEthernet0/1.10
 encapsulation dot1Q 10
 ip address 10.10.10.2 255.255.255.0
 ip access-group ACL-VLAN10-IN in
 standby 10 ip 10.10.10.1
 standby 10 priority 110
 standby 10 preempt
 standby 10 track 1 decrement 20
!
interface GigabitEthernet0/1.20
 encapsulation dot1Q 20
 ip address 10.10.20.2 255.255.255.0
 ip access-group ACL-VLAN20-IN in
 standby 20 ip 10.10.20.1
 standby 20 priority 110
 standby 20 preempt
 standby 20 track 1 decrement 20
!
interface GigabitEthernet0/1.30
 encapsulation dot1Q 30
 ip address 10.10.30.2 255.255.255.0
 ip access-group ACL-VLAN30-IN in
 standby 30 ip 10.10.30.1
 standby 30 priority 110
 standby 30 preempt
 standby 30 track 1 decrement 20
!
interface GigabitEthernet0/1.40
 encapsulation dot1Q 40
 ip address 10.10.40.2 255.255.255.0
 ip access-group ACL-VLAN40-IN in
 standby 40 ip 10.10.40.1
 standby 40 priority 110
 standby 40 preempt
 standby 40 track 1 decrement 20
!
interface GigabitEthernet0/1.50
 encapsulation dot1Q 50
 ip address 10.10.50.2 255.255.255.0
 ip access-group ACL-VLAN50-IN in
 standby 50 ip 10.10.50.1
 standby 50 priority 110
 standby 50 preempt
 standby 50 track 1 decrement 20
!
interface GigabitEthernet0/1.60
 encapsulation dot1Q 60
 ip address 10.10.60.2 255.255.255.0
 ip access-group ACL-VLAN60-IN in
 standby 60 ip 10.10.60.1
 standby 60 priority 110
 standby 60 preempt
 standby 60 track 1 decrement 20
!
interface GigabitEthernet0/1.99
 encapsulation dot1Q 99
 ip address 10.10.99.2 255.255.255.0
 standby 99 ip 10.10.99.1
 standby 99 priority 110
 standby 99 preempt
 standby 99 track 1 decrement 20
!
ip dhcp excluded-address 10.10.10.1 10.10.10.10
ip dhcp excluded-address 10.10.20.1 10.10.20.10
ip dhcp excluded-address 10.10.30.1 10.10.30.10
ip dhcp excluded-address 10.10.40.1 10.10.40.10
ip dhcp excluded-address 10.10.50.1 10.10.50.10
!
ip dhcp pool VLAN10-ADMIN
 network 10.10.10.0 255.255.255.0
 default-router 10.10.10.1
 dns-server 8.8.8.8
!
ip dhcp pool VLAN20-HR
 network 10.10.20.0 255.255.255.0
 default-router 10.10.20.1
 dns-server 8.8.8.8
!
ip dhcp pool VLAN30-FINANCE
 network 10.10.30.0 255.255.255.0
 default-router 10.10.30.1
 dns-server 8.8.8.8
!
ip dhcp pool VLAN40-DEV
 network 10.10.40.0 255.255.255.0
 default-router 10.10.40.1
 dns-server 8.8.8.8
!
ip dhcp pool VLAN50-TRAINING
 network 10.10.50.0 255.255.255.0
 default-router 10.10.50.1
 dns-server 8.8.8.8
!
ip access-list extended ACL-VLAN10-IN
 deny ip any 10.10.99.0 0.0.0.255
 permit ip any any
!
ip access-list extended ACL-VLAN20-IN
 deny ip any 10.10.99.0 0.0.0.255
 permit ip any any
!
ip access-list extended ACL-VLAN30-IN
 deny ip any 10.10.99.0 0.0.0.255
 deny ip any 10.10.40.0 0.0.0.255
 permit ip any any
!
ip access-list extended ACL-VLAN40-IN
 deny ip any 10.10.99.0 0.0.0.255
 deny ip any 10.10.30.0 0.0.0.255
 permit ip any any
!
ip access-list extended ACL-VLAN50-IN
 deny ip any 10.10.30.0 0.0.0.255
 deny ip any 10.10.40.0 0.0.0.255
 deny ip any 10.10.60.0 0.0.0.255
 deny ip any 10.10.99.0 0.0.0.255
 permit ip any any
!
ip access-list extended ACL-VLAN60-IN
 deny ip any 10.10.99.0 0.0.0.255
 permit ip any any
!
ip access-list standard NOC-MGMT-ONLY
 permit 10.10.99.0 0.0.0.255
!
ntp master 3
logging host 10.10.99.20
logging trap informational
!
line vty 0 4
 transport input ssh
 login local
 access-class NOC-MGMT-ONLY in
line con 0
 logging synchronous
 exec-timeout 5 0
!
banner motd ^Authorized access only. All activity is logged.^
end

### R-CAMP2

hostname R-CAMP2
no ip domain-lookup
ip domain-name campus.local
enable secret Cisco12345
service password-encryption
username netadmin privilege 15 secret Cisco12345
crypto key generate rsa modulus 1024
ip ssh version 2
!
track 1 interface GigabitEthernet0/1 line-protocol
!
interface GigabitEthernet0/0
 description Uplink to campus WAN
 ip address dhcp
 no shutdown
!
interface GigabitEthernet0/1
 description Trunk to DSW2
 no ip address
 no shutdown
!
interface GigabitEthernet0/1.10
 encapsulation dot1Q 10
 ip address 10.10.10.3 255.255.255.0
 ip access-group ACL-VLAN10-IN in
 standby 10 ip 10.10.10.1
 standby 10 priority 90
 standby 10 preempt
 standby 10 track 1 decrement 20
!
interface GigabitEthernet0/1.20
 encapsulation dot1Q 20
 ip address 10.10.20.3 255.255.255.0
 ip access-group ACL-VLAN20-IN in
 standby 20 ip 10.10.20.1
 standby 20 priority 90
 standby 20 preempt
 standby 20 track 1 decrement 20
!
interface GigabitEthernet0/1.30
 encapsulation dot1Q 30
 ip address 10.10.30.3 255.255.255.0
 ip access-group ACL-VLAN30-IN in
 standby 30 ip 10.10.30.1
 standby 30 priority 90
 standby 30 preempt
 standby 30 track 1 decrement 20
!
interface GigabitEthernet0/1.40
 encapsulation dot1Q 40
 ip address 10.10.40.3 255.255.255.0
 ip access-group ACL-VLAN40-IN in
 standby 40 ip 10.10.40.1
 standby 40 priority 90
 standby 40 preempt
 standby 40 track 1 decrement 20
!
interface GigabitEthernet0/1.50
 encapsulation dot1Q 50
 ip address 10.10.50.3 255.255.255.0
 ip access-group ACL-VLAN50-IN in
 standby 50 ip 10.10.50.1
 standby 50 priority 90
 standby 50 preempt
 standby 50 track 1 decrement 20
!
interface GigabitEthernet0/1.60
 encapsulation dot1Q 60
 ip address 10.10.60.3 255.255.255.0
 ip access-group ACL-VLAN60-IN in
 standby 60 ip 10.10.60.1
 standby 60 priority 90
 standby 60 preempt
 standby 60 track 1 decrement 20
!
interface GigabitEthernet0/1.99
 encapsulation dot1Q 99
 ip address 10.10.99.3 255.255.255.0
 standby 99 ip 10.10.99.1
 standby 99 priority 90
 standby 99 preempt
 standby 99 track 1 decrement 20
!
ip access-list extended ACL-VLAN10-IN
 deny ip any 10.10.99.0 0.0.0.255
 permit ip any any
!
ip access-list extended ACL-VLAN20-IN
 deny ip any 10.10.99.0 0.0.0.255
 permit ip any any
!
ip access-list extended ACL-VLAN30-IN
 deny ip any 10.10.99.0 0.0.0.255
 deny ip any 10.10.40.0 0.0.0.255
 permit ip any any
!
ip access-list extended ACL-VLAN40-IN
 deny ip any 10.10.99.0 0.0.0.255
 deny ip any 10.10.30.0 0.0.0.255
 permit ip any any
!
ip access-list extended ACL-VLAN50-IN
 deny ip any 10.10.30.0 0.0.0.255
 deny ip any 10.10.40.0 0.0.0.255
 deny ip any 10.10.60.0 0.0.0.255
 deny ip any 10.10.99.0 0.0.0.255
 permit ip any any
!
ip access-list extended ACL-VLAN60-IN
 deny ip any 10.10.99.0 0.0.0.255
 permit ip any any
!
ip access-list standard NOC-MGMT-ONLY
 permit 10.10.99.0 0.0.0.255
!
ntp server 10.10.10.2
logging host 10.10.99.20
logging trap informational
!
line vty 0 4
 transport input ssh
 login local
 access-class NOC-MGMT-ONLY in
line con 0
 logging synchronous
 exec-timeout 5 0
end

### DSW1

hostname DSW1
enable secret Cisco12345
service password-encryption
username netadmin privilege 15 secret Cisco12345
ip domain-name campus.local
crypto key generate rsa modulus 1024
ip ssh version 2
!
vlan 10
 name ADMIN
vlan 20
 name HR
vlan 30
 name FINANCE
vlan 40
 name DEVELOPMENT
vlan 50
 name TRAINING-LAB
vlan 60
 name SERVER-ROOM
vlan 99
 name NETWORK-MGMT
vlan 999
 name NATIVE-UNUSED
!
spanning-tree mode rapid-pvst
spanning-tree vlan 10,30,50,99 root primary
spanning-tree vlan 20,40,60 root secondary
!
ip dhcp snooping vlan 10,20,30,40,50,60
ip dhcp snooping database flash:dhcp-snoop-db
ip arp inspection vlan 10,20,30,40,50,60
!
interface range GigabitEthernet0/2-3
 description LACP to DSW2
 switchport trunk encapsulation dot1q
 switchport mode trunk
 switchport trunk native vlan 999
 switchport trunk allowed vlan 10,20,30,40,50,60,99
 channel-group 1 mode active
 ip dhcp snooping trust
 ip arp inspection trust
!
interface Port-channel1
 switchport trunk encapsulation dot1q
 switchport mode trunk
 switchport trunk native vlan 999
 switchport trunk allowed vlan 10,20,30,40,50,60,99
 udld port aggressive
!
interface GigabitEthernet0/1
 description Trunk to R-CAMP1
 switchport trunk encapsulation dot1q
 switchport mode trunk
 switchport trunk native vlan 999
 switchport trunk allowed vlan 10,20,30,40,50,60,99
!
interface range GigabitEthernet0/4-6
 description Trunks to ASW-Floor1/2/3
 switchport trunk encapsulation dot1q
 switchport mode trunk
 switchport trunk native vlan 999
 switchport trunk allowed vlan 10,20,30,40,50,60,99
 spanning-tree guard root
 ip dhcp snooping trust
 ip arp inspection trust
!
interface Vlan99
 ip address 10.10.99.4 255.255.255.0
 no shutdown
!
ip default-gateway 10.10.99.1
ntp server 10.10.10.2
logging host 10.10.99.20
logging trap informational
!
ip access-list standard NOC-MGMT-ONLY
 permit 10.10.99.0 0.0.0.255
!
line vty 0 4
 transport input ssh
 login local
 access-class NOC-MGMT-ONLY in
line con 0
 logging synchronous
 exec-timeout 5 0
end

### DSW2

hostname DSW2
enable secret Cisco12345
service password-encryption
username netadmin privilege 15 secret Cisco12345
ip domain-name campus.local
crypto key generate rsa modulus 1024
ip ssh version 2
!
vlan 10
 name ADMIN
vlan 20
 name HR
vlan 30
 name FINANCE
vlan 40
 name DEVELOPMENT
vlan 50
 name TRAINING-LAB
vlan 60
 name SERVER-ROOM
vlan 99
 name NETWORK-MGMT
vlan 999
 name NATIVE-UNUSED
!
spanning-tree mode rapid-pvst
spanning-tree vlan 20,40,60 root primary
spanning-tree vlan 10,30,50,99 root secondary
!
ip dhcp snooping vlan 10,20,30,40,50,60
ip dhcp snooping database flash:dhcp-snoop-db
ip arp inspection vlan 10,20,30,40,50,60
!
interface range GigabitEthernet0/2-3
 description LACP to DSW1
 switchport trunk encapsulation dot1q
 switchport mode trunk
 switchport trunk native vlan 999
 switchport trunk allowed vlan 10,20,30,40,50,60,99
 channel-group 1 mode active
 ip dhcp snooping trust
 ip arp inspection trust
!
interface Port-channel1
 switchport trunk encapsulation dot1q
 switchport mode trunk
 switchport trunk native vlan 999
 switchport trunk allowed vlan 10,20,30,40,50,60,99
 udld port aggressive
!
interface GigabitEthernet0/1
 description Trunk to R-CAMP2
 switchport trunk encapsulation dot1q
 switchport mode trunk
 switchport trunk native vlan 999
 switchport trunk allowed vlan 10,20,30,40,50,60,99
!
interface range GigabitEthernet0/4-6
 description Trunks to ASW-Floor1/2/3
 switchport trunk encapsulation dot1q
 switchport mode trunk
 switchport trunk native vlan 999
 switchport trunk allowed vlan 10,20,30,40,50,60,99
 spanning-tree guard root
 ip dhcp snooping trust
 ip arp inspection trust
!
interface Vlan99
 ip address 10.10.99.5 255.255.255.0
 no shutdown
!
ip default-gateway 10.10.99.1
ntp server 10.10.10.2
logging host 10.10.99.20
logging trap informational
!
ip access-list standard NOC-MGMT-ONLY
 permit 10.10.99.0 0.0.0.255
!
line vty 0 4
 transport input ssh
 login local
 access-class NOC-MGMT-ONLY in
line con 0
 logging synchronous
 exec-timeout 5 0
end

### ASW-Floor1 (Admin + HR)

hostname ASW-F1
enable secret Cisco12345
service password-encryption
username netadmin privilege 15 secret Cisco12345
ip domain-name campus.local
crypto key generate rsa modulus 1024
ip ssh version 2
!
vlan 10
 name ADMIN
vlan 20
 name HR
vlan 99
 name NETWORK-MGMT
vlan 999
 name NATIVE-UNUSED
!
spanning-tree mode rapid-pvst
!
interface range GigabitEthernet0/1-2
 description Uplinks — Gi0/1 to DSW1, Gi0/2 to DSW2
 switchport trunk encapsulation dot1q
 switchport mode trunk
 switchport trunk native vlan 999
 switchport trunk allowed vlan 10,20,99
!
interface range FastEthernet0/1-10
 description Admin PCs
 switchport mode access
 switchport access vlan 10
 switchport port-security
 switchport port-security maximum 2
 switchport port-security violation restrict
 switchport port-security mac-address sticky
 spanning-tree portfast
 spanning-tree bpduguard enable
!
interface range FastEthernet0/11-20
 description HR PCs and shared printer
 switchport mode access
 switchport access vlan 20
 switchport port-security
 switchport port-security maximum 2
 switchport port-security violation restrict
 switchport port-security mac-address sticky
 spanning-tree portfast
 spanning-tree bpduguard enable
!
interface Vlan99
 ip address 10.10.99.6 255.255.255.0
 no shutdown
!
ip default-gateway 10.10.99.1
ntp server 10.10.10.2
logging host 10.10.99.20
!
ip access-list standard NOC-MGMT-ONLY
 permit 10.10.99.0 0.0.0.255
!
line vty 0 4
 transport input ssh
 login local
 access-class NOC-MGMT-ONLY in
line con 0
 logging synchronous
 exec-timeout 5 0
end

### ASW-Floor2 (Finance + Development + Server Room)

hostname ASW-F2
enable secret Cisco12345
service password-encryption
username netadmin privilege 15 secret Cisco12345
ip domain-name campus.local
crypto key generate rsa modulus 1024
ip ssh version 2
!
vlan 30
 name FINANCE
vlan 40
 name DEVELOPMENT
vlan 60
 name SERVER-ROOM
vlan 99
 name NETWORK-MGMT
vlan 999
 name NATIVE-UNUSED
!
spanning-tree mode rapid-pvst
!
interface range GigabitEthernet0/1-2
 description Uplinks — Gi0/1 to DSW1, Gi0/2 to DSW2
 switchport trunk encapsulation dot1q
 switchport mode trunk
 switchport trunk native vlan 999
 switchport trunk allowed vlan 30,40,60,99
!
interface range FastEthernet0/1-10
 description Finance PCs
 switchport mode access
 switchport access vlan 30
 switchport port-security
 switchport port-security maximum 2
 switchport port-security violation restrict
 switchport port-security mac-address sticky
 spanning-tree portfast
 spanning-tree bpduguard enable
!
interface range FastEthernet0/11-20
 description Development PCs
 switchport mode access
 switchport access vlan 40
 switchport port-security
 switchport port-security maximum 2
 switchport port-security violation restrict
 switchport port-security mac-address sticky
 spanning-tree portfast
 spanning-tree bpduguard enable
!
interface range FastEthernet0/21-23
 description SRV-DHCP / SRV-FIN / SRV-DEV
 switchport mode access
 switchport access vlan 60
 switchport port-security
 switchport port-security maximum 1
 switchport port-security violation restrict
 switchport port-security mac-address sticky
 spanning-tree portfast
 spanning-tree bpduguard enable
!
interface Vlan99
 ip address 10.10.99.7 255.255.255.0
 no shutdown
!
ip default-gateway 10.10.99.1
ntp server 10.10.10.2
logging host 10.10.99.20
!
ip access-list standard NOC-MGMT-ONLY
 permit 10.10.99.0 0.0.0.255
!
line vty 0 4
 transport input ssh
 login local
 access-class NOC-MGMT-ONLY in
line con 0
 logging synchronous
 exec-timeout 5 0
end

### ASW-Floor3 (Training Lab + NOC)

hostname ASW-F3
enable secret Cisco12345
service password-encryption
username netadmin privilege 15 secret Cisco12345
ip domain-name campus.local
crypto key generate rsa modulus 1024
ip ssh version 2
!
vlan 50
 name TRAINING-LAB
vlan 99
 name NETWORK-MGMT
vlan 999
 name NATIVE-UNUSED
!
spanning-tree mode rapid-pvst
!
interface range GigabitEthernet0/1-2
 description Uplinks — Gi0/1 to DSW1, Gi0/2 to DSW2
 switchport trunk encapsulation dot1q
 switchport mode trunk
 switchport trunk native vlan 999
 switchport trunk allowed vlan 50,99
!
interface range FastEthernet0/1-10
 description Training Lab PCs
 switchport mode access
 switchport access vlan 50
 switchport port-security
 switchport port-security maximum 2
 switchport port-security violation restrict
 switchport port-security mac-address sticky
 spanning-tree portfast
 spanning-tree bpduguard enable
!
interface FastEthernet0/11
 description NOC-PC — SSH admin only
 switchport mode access
 switchport access vlan 99
 switchport port-security
 switchport port-security maximum 1
 switchport port-security violation restrict
 switchport port-security mac-address sticky
 spanning-tree portfast
 spanning-tree bpduguard enable
!
interface Vlan99
 ip address 10.10.99.8 255.255.255.0
 no shutdown
!
ip default-gateway 10.10.99.1
ntp server 10.10.10.2
logging host 10.10.99.20
!
ip access-list standard NOC-MGMT-ONLY
 permit 10.10.99.0 0.0.0.255
!
line vty 0 4
 transport input ssh
 login local
 access-class NOC-MGMT-ONLY in
line con 0
 logging synchronous
 exec-timeout 5 0
end

# Phase-by-phase implementation process:

Each phase maps directly to a section of the configs above. Complete and verify one phase fully before starting the next — this is what your before/after evidence and incident report should be structured around.

## Phase 1 — VLAN design and creation

- Create VLANs 10, 20, 30, 40, 50, 60, 99, and 999 on DSW1, DSW2, and all three ASWs (Section 11).

- Name every VLAN to match Section 4 so show vlan brief output is self-documenting.

Verify:  show vlan brief

·            show interfaces status

## Phase 2 — Build access and distribution switching layer

- Cable each ASW with two uplinks — one to DSW1, one to DSW2.

- Cable DSW1 and DSW2 together with two links for the future EtherChannel.

- Bring up all access ports and confirm link lights / status before applying VLAN assignments.

Verify:   show interfaces status  ·

show cdp neighbors

## Phase 3 — Configure trunks, allowed VLANs, and native VLAN

- Apply the trunk blocks from Section 11 on every inter-switch link.

- Set native VLAN 999 on every trunk — confirm it matches on both ends of each link to avoid a native VLAN mismatch.

- Restrict the allowed VLAN list on each ASW uplink to only the VLANs that floor actually uses.

Verify:  show interfaces trunk  ·

show interfaces <int> switchport

## Phase 4 — Configure EtherChannel between distribution switches

- Apply channel-group 1 mode active on both members on DSW1 and DSW2 (LACP active/active).

- Confirm the port-channel forms before moving on — a common fault here is one side set to passive.

Verify: show etherchannel summary

·            show interfaces port-channel 1

## Phase 5 — Set STP root primary/secondary per VLAN group and PortFast

- Apply the split root configuration from Section 3.2 on DSW1 and DSW2.

- Enable PortFast and BPDU guard on every access-facing port on all three ASWs.

- Enable root guard on DSW1/DSW2 ports facing the access layer.

Verify: show spanning-tree vlan <id>

·  show spanning-tree summary

## Phase 6 — Configure inter-VLAN routing with HSRP

- Apply router-on-a-stick sub-interfaces and HSRP groups from Section 11.1–11.2 on both routers.

- Add track 1 on the uplink interface on both routers so HSRP reacts to link failure, not just router failure.

- Configure DHCP pools for the five user VLANs.

Verify: show standby brief  ·

show ip interface brief  ·

show ip dhcp binding

## Phase 7 — Apply port security, DHCP snooping, ARP inspection, and ACLs

- Apply port security on every access port (Section 11.5–11.7).

- Enable DHCP snooping and Dynamic ARP Inspection on DSW1, DSW2, and trust the uplink ports (Section 11.3–11.4).

- Apply the ACL-VLANxx-IN access lists on both routers (Section 10) and the NOC-MGMT-ONLY access-class on every device's VTY lines.

Verify: show port-security

·  show ip dhcp snooping

·              show ip arp inspection  ·

show access-lists

## Phase 8 — Redundancy, access-control, and documentation testing

- Shut down each distribution switch in turn and confirm every access switch keeps connectivity through the other.

- Shut down R-CAMP1's uplink and confirm HSRP fails over to R-CAMP2 within a few seconds.

- Test every deny/allow pair in the Section 10 matrix with ping and document pass/fail.

- Capture before/after show-command output for every phase and write up any faults encountered in the incident report.

Verify: show standby brief   ·

show spanning-tree vlan <id>

·            ping / traceroute across VLAN pairs