REDUNDANT MULTI-FLOOR ENTERPRISE CAMPUS NETWORK

IP Addressing & Network Plan

# 1. Network Design Overview

The enterprise campus uses VLAN-based segmentation at the access layer, Layer-3 gateway interfaces on DSW1 and DSW2, HSRP for first-hop gateway redundancy, and routed point-to-point links between the distribution switches and campus routers. DSW1 and DSW2 are connected by LACP EtherChannel Po1, while each access switch has redundant 802.1Q trunk uplinks to both distribution switches.

# 2. VLAN and LAN IP Addressing Plan

| VLAN | Segment / Purpose | Network | Mask | HSRP Virtual Gateway | DSW1 SVI | DSW2 SVI |
| --- | --- | --- | --- | --- | --- | --- |
| 10 | ADMIN | 10.10.10.0/24 | 255.255.255.0 | 10.10.10.1 | 10.10.10.2 | 10.10.10.3 |
| 20 | HR | 10.10.20.0/24 | 255.255.255.0 | 10.10.20.1 | 10.10.20.2 | 10.10.20.3 |
| 30 | FINANCE | 10.10.30.0/24 | 255.255.255.0 | 10.10.30.1 | 10.10.30.2 | 10.10.30.3 |
| 40 | DEVELOPMENT | 10.10.40.0/24 | 255.255.255.0 | 10.10.40.1 | 10.10.40.2 | 10.10.40.3 |
| 50 | TRAINING | 10.10.50.0/24 | 255.255.255.0 | 10.10.50.1 | 10.10.50.2 | 10.10.50.3 |
| 60 | SERVER | 10.10.60.0/24 | 255.255.255.0 | 10.10.60.1 | 10.10.60.2 | 10.10.60.3 |
| 99 | MANAGEMENT | 192.168.99.0/24 | 255.255.255.0 | 192.168.99.1 | 192.168.99.2 | 192.168.99.3 |

Default gateway for end devices in each VLAN is the HSRP virtual IP address. DSW1 is currently configured with HSRP priority 110 and DSW2 with priority 100, so DSW1 is the preferred active gateway for all listed VLANs.

# 3. Routed Point-to-Point Network Plan

| Link | Device / Interface | IP Address | Peer | Peer IP | Network |
| --- | --- | --- | --- | --- | --- |
| R-CAMP1 ↔ DSW1 | R-CAMP1 G0/0 | 10.255.1.1/30 | DSW1 Fa0/4 | 10.255.1.2/30 | 10.255.1.0/30 |
| R-CAMP1 ↔ DSW2 | R-CAMP1 G0/1 | 10.255.1.5/30 | DSW2 Fa0/5 | 10.255.1.6/30 | 10.255.1.4/30 |
| R-CAMP2 ↔ DSW2 | R-CAMP2 G0/0 | 10.255.2.1/30 | DSW2 Fa0/4 | 10.255.2.2/30 | 10.255.2.0/30 |
| R-CAMP2 ↔ DSW1 | R-CAMP2 G0/1 | 10.255.2.5/30 | DSW1 Fa0/5 | 10.255.2.6/30 | 10.255.2.4/30 |

# 4. Device and Interface Summary

| Device | Role | Relevant Interfaces / IPs | Function |
| --- | --- | --- | --- |
| DSW1 | Distribution / Layer 3 | SVIs 10–60, 99; Fa0/4 10.255.1.2; Fa0/5 10.255.2.6 | Inter-VLAN gateways, HSRP, routed uplinks, STP root preference |
| DSW2 | Distribution / Layer 3 | SVIs 10–60, 99; Fa0/4 10.255.2.2; Fa0/5 10.255.1.6 | Backup HSRP gateway, routed uplinks, STP secondary |
| R-CAMP1 | Campus Router 1 | G0/0 10.255.1.1; G0/1 10.255.1.5 | Routed connectivity toward DSW pair |
| R-CAMP2 | Campus Router 2 | G0/0 10.255.2.1; G0/1 10.255.2.5 | Routed connectivity toward DSW pair |
| ASW1 | Access Switch | Management VLAN 99 planned; dual trunk uplinks | ADMIN/HR access connectivity |
| ASW2 | Access Switch | Management VLAN 99 planned; dual trunk uplinks | FINANCE/DEVELOPMENT/SERVER access connectivity |
| ASW3 | Access Switch | Management VLAN 99 planned; dual trunk uplinks | TRAINING/MANAGEMENT access connectivity |

# 5. HSRP Plan

| VLAN | Virtual IP | DSW1 | DSW1 Priority | DSW2 | DSW2 Priority | Expected Active |
| --- | --- | --- | --- | --- | --- | --- |
| 10 | 10.10.10.1 | 10.10.10.2 | 110 | 10.10.10.3 | 100 | DSW1 |
| 20 | 10.10.20.1 | 10.10.20.2 | 110 | 10.10.20.3 | 100 | DSW1 |
| 30 | 10.10.30.1 | 10.10.30.2 | 110 | 10.10.30.3 | 100 | DSW1 |
| 40 | 10.10.40.1 | 10.10.40.2 | 110 | 10.10.40.3 | 100 | DSW1 |
| 50 | 10.10.50.1 | 10.10.50.2 | 110 | 10.10.50.3 | 100 | DSW1 |
| 60 | 10.10.60.1 | 10.10.60.2 | 110 | 10.10.60.3 | 100 | DSW1 |
| 99 | 192.168.99.1 | 192.168.99.2 | 110 | 192.168.99.3 | 100 | DSW1 |

# 6. Routing / Inter-VLAN Design

Inter-VLAN routing is performed on DSW1 and DSW2 using SVIs, not Router-on-a-Stick. Because ip routing is enabled on the multilayer switches, each SVI network is directly connected to the corresponding distribution switch. End devices use the HSRP virtual IP as their default gateway.

OSPF can be added between R-CAMP1, R-CAMP2, DSW1, and DSW2 to dynamically advertise the campus networks and provide redundant routed paths. The current addressing plan reserves /30 networks for these point-to-point routed links.

# 7. Access Switch VLAN Assignment

| Access Switch | Ports / Users | VLAN | Network |
| --- | --- | --- | --- |
| ASW1 | Fa0/1–Fa0/10 ADMIN users | 10 | 10.10.10.0/24 |
| ASW1 | Fa0/11–Fa0/15 HR users | 20 | 10.10.20.0/24 |
| ASW2 | Fa0/1–Fa0/10 FINANCE users | 30 | 10.10.30.0/24 |
| ASW2 | Fa0/11–Fa0/20 DEVELOPMENT users | 40 | 10.10.40.0/24 |
| ASW2 | Fa0/21–Fa0/23 SERVER ports | 60 | 10.10.60.0/24 |
| ASW3 | Fa0/1–Fa0/10 TRAINING users | 50 | 10.10.50.0/24 |
| ASW3 | Fa0/11 MANAGEMENT | 99 | 192.168.99.0/24 |

# 8. Trunk and Redundancy Plan

Each access switch uses GigabitEthernet0/1 as the uplink toward DSW1 and GigabitEthernet0/2 as the uplink toward DSW2. Both are 802.1Q trunks allowing VLANs 10, 20, 30, 40, 50, 60 and 99. RSTP/PVST+ blocks the redundant path where necessary; the observed STP state shows Gi0/1 as Root FWD and Gi0/2 as Alternate/Blocked for the sampled VLANs.

# 9. Verification Commands

show ip interface brief

show ip route

show standby brief

show spanning-tree vlan 10

show etherchannel summary

show interfaces trunk

show vlan brief

show cdp neighbors

ping 10.255.1.1

ping 10.255.1.5

ping 10.255.2.1

ping 10.255.2.5

# 10. Important Implementation Notes

The HSRP virtual IP (.1) is the default gateway for clients; do not configure clients to use the DSW physical IPs (.2 or .3) as their normal gateway.

The four /30 routed links are point-to-point networks and should not be used for end-user devices.

The current topology uses Layer-3 SVIs on the distribution switches, so a separate Router-on-a-Stick configuration is not required for inter-VLAN routing.

Before adding OSPF, verify all four routed links are correctly addressed and reachable.

For DHCP, the DHCP server can reside in VLAN 60; use ip helper-address on each client VLAN SVI if the DHCP server is not local to that VLAN.

# 11. Addressing Quick Reference

LAN summary: 10.10.10.0/24 through 10.10.60.0/24 and 10.10.99.0/24. Transit summary: 10.255.1.0/30, 10.255.1.4/30, 10.255.2.0/30, and 10.255.2.4/30.