REDUNDANT MULTI-FLOOR   ENTERPRISE CAMPUS NETWORK

Project Documentation & Configuration Report

| Document Control | Value |
| --- | --- |
| Project | Redundant Multi-Floor Enterprise Campus Network |
| Platform | Cisco Packet Tracer |
| Architecture | Two-tier campus: Distribution + Access, with dual campus routers |
| Distribution | DSW1, DSW2 |
| Access | ASW1, ASW2, ASW3 |
| Routers | R-CAMP1, R-CAMP2 |
| Management | Dedicated VLAN 99 + static Management PC (10.10.99.10) |
| Completion Status | All stages complete: VLANs, HSRP, OSPF, EtherChannel, DHCP, SSH, ACLs, testing |

# Table of Contents

1. Executive Summary

2. Project Objectives

3. Project Flow Chart

4. Network Architecture

5. Device Inventory

6. VLAN and IP Addressing Plan

7. Routed Link Addressing

8. Layer-2 Configuration

9. HSRP Gateway Redundancy

10. OSPF Dynamic Routing

11. DHCP Services

12. EtherChannel and Trunking

13. SSH Management

14. Management PC

15. Access Control and Security

15.1 TRAINING-to-SERVER ACL

15.2 SSH Management ACL

16. Verification and Testing

17. SSH Testing

18. Troubleshooting Record

18.1 ACL Existed but Was Initially Not Shown as Applied to VLAN 50

18.2 ASW1 VTY Configuration

19. Final Network Status

20. Conclusion

Appendix A — Management Addressing

Appendix B — Credentials

Appendix C — Important Commands

Appendix D — Project Change Log

# 1. Executive Summary

This project implements a redundant multi-floor enterprise campus network in Cisco Packet Tracer. The design uses two campus routers, two Layer-3 distribution switches and three access switches. Departmental traffic is segmented into VLANs, default-gateway redundancy is provided by HSRP, dynamic routing is provided by OSPF, and resilient Layer-2 connectivity between the distribution switches is provided by LACP EtherChannel. DHCP provides automatic addressing for user VLANs.

Security and management are implemented using SSH version 2, local authentication, a Management VLAN, a static Management PC, and ACLs. The TRAINING VLAN is restricted from reaching the SERVER VLAN, while SSH management is restricted to the Management VLAN. Verification commands, ACL counters, connectivity tests and troubleshooting results were recorded throughout the implementation.

# 2. Project Objectives

- Design a scalable and redundant enterprise campus network.

- Segment departments using VLANs.

- Provide redundant default gateways using HSRP.

- Implement OSPF dynamic routing.

- Provide DHCP services for required VLANs.

- Implement LACP EtherChannel and trunking.

- Secure remote administration using SSH version 2.

- Use ACLs to control management access and inter-VLAN traffic.

- Provide a dedicated Management VLAN and static Management PC.

- Verify, troubleshoot and document the complete network.

# 3. Project Flow Chart

The project follows a structured implementation process. Each stage is completed and verified before proceeding to the next stage.

| 01 | REQUIREMENT ANALYSIS | Identify business, security, redundancy and connectivity requirements. |
| --- | --- | --- |
| ▼ |  |  |
| 02 | NETWORK TOPOLOGY DESIGN | Design the hierarchical campus topology and device interconnections. |
| ▼ |  |  |
| 03 | DEVICE CONFIGURATION | Configure routers, distribution switches and access switches. |
| ▼ |  |  |
| 04 | VLAN & IP ADDRESSING | Create departmental VLANs and implement the IP addressing plan. |
| ▼ |  |  |
| 05 | REDUNDANCY IMPLEMENTATION | Configure HSRP gateway redundancy and LACP EtherChannel. |
| ▼ |  |  |
| 06 | OSPF ROUTING | Establish dynamic routing between routers and Layer-3 switches. |
| ▼ |  |  |
| 07 | DHCP & SSH SECURITY | Configure DHCP services and secure remote administration using SSHv2. |
| ▼ |  |  |
| 08 | ACL & NETWORK SECURITY | Restrict unauthorized management and TRAINING-to-SERVER traffic. |
| ▼ |  |  |
| 09 | TESTING & VERIFICATION | Verify connectivity, routing, HSRP, EtherChannel, SSH and ACL operation. |
| ▼ |  |  |
| 10 | FINAL SECURE NETWORK | Complete troubleshooting, save configurations and validate the final network. |

Figure 3: Project Implementation Flowchart

Flow summary: Requirement Analysis → Topology Design → Device Configuration → VLAN & IP Configuration → HSRP/OSPF/EtherChannel → DHCP & SSH → ACL Security → Testing → Troubleshooting → Final Verification → Secure & Redundant Network.

# 4. Network Architecture

The architecture follows a hierarchical enterprise model. The distribution layer performs Layer-3 routing and gateway functions. Access switches provide endpoint connectivity. Redundant links and gateway protocols improve availability.

# 5. Device Inventory

| Device | Role | Management / Key IP | Functions |
| --- | --- | --- | --- |
| R-CAMP1 | Campus Router | 10.255.1.1 / 10.255.1.5 | OSPF, SSH |
| R-CAMP2 | Campus Router | 10.255.2.1 / 10.255.2.5 | OSPF, SSH |
| DSW1 | Layer-3 Distribution | 10.10.99.2 | SVIs, HSRP, OSPF, DHCP, ACL, LACP, SSH |
| DSW2 | Layer-3 Distribution | 10.10.99.3 | SVIs, HSRP, OSPF, ACL, LACP, SSH |
| ASW1 | Access Switch | 10.10.99.11 | Access VLANs, trunks, SSH |
| ASW2 | Access Switch | 10.10.99.12 | Access VLANs, trunks, SSH |
| ASW3 | Access Switch | 10.10.99.13 | Access VLANs, trunks, SSH |
| Management PC | Management workstation | 10.10.99.10 | Static IP, SSH administration |

# 6. VLAN and IP Addressing Plan

| VLAN | Name | Network | HSRP VIP | DSW1 | DSW2 |
| --- | --- | --- | --- | --- | --- |
| 10 | ADMIN | 10.10.10.0/24 | 10.10.10.1 | 10.10.10.2 | 10.10.10.3 |
| 20 | HR | 10.10.20.0/24 | 10.10.20.1 | 10.10.20.2 | 10.10.20.3 |
| 30 | FINANCE | 10.10.30.0/24 | 10.10.30.1 | 10.10.30.2 | 10.10.30.3 |
| 40 | DEVELOPMENT | 10.10.40.0/24 | 10.10.40.1 | 10.10.40.2 | 10.10.40.3 |
| 50 | TRAINING | 10.10.50.0/24 | 10.10.50.1 | 10.10.50.2 | 10.10.50.3 |
| 60 | SERVER | 10.10.60.0/24 | 10.10.60.1 | 10.10.60.2 | 10.10.60.3 |
| 99 | MANAGEMENT | 10.10.99.0/24 | 10.10.99.1 | 10.10.99.2 | 10.10.99.3 |

# 7. Routed Link Addressing

| Link | Network | Endpoint A | Endpoint B |
| --- | --- | --- | --- |
| R-CAMP1–DSW1 | 10.255.1.0/30 | R-CAMP1 G0/0 = 10.255.1.1 | DSW1 Fa0/4 = 10.255.1.2 |
| R-CAMP1–DSW2 | 10.255.1.4/30 | R-CAMP1 G0/1 = 10.255.1.5 | DSW2 Fa0/5 = 10.255.1.6 |
| R-CAMP2–DSW2 | 10.255.2.0/30 | R-CAMP2 G0/0 = 10.255.2.1 | DSW2 Fa0/4 = 10.255.2.2 |
| R-CAMP2–DSW1 | 10.255.2.4/30 | R-CAMP2 G0/1 = 10.255.2.5 | DSW1 Fa0/5 = 10.255.2.6 |

# 8. Layer-2 Configuration

VLANs 10, 20, 30, 40, 50, 60 and 99 were created and assigned according to department and function. VLAN 99 is reserved for network management. Trunk links carry the required VLANs between network layers. Rapid-PVST is enabled for loop prevention and convergence.

VLAN 10 ADMIN
VLAN 20 HR
VLAN 30 FINANCE
VLAN 40 DEVELOPMENT
VLAN 50 TRAINING
VLAN 60 SERVER
VLAN 99 MANAGEMENT

# 9. HSRP Gateway Redundancy

DSW1 and DSW2 provide redundant Layer-3 gateways. The virtual gateway for each VLAN uses the .1 address. DSW1 uses the .2 address and DSW2 uses the .3 address. DSW1 is configured with HSRP priority 110 and preemption, making it the preferred gateway under normal conditions.

interface Vlan50
description TRAINING-GATEWAY
ip address 10.10.50.2 255.255.255.0
standby 50 ip 10.10.50.1
standby 50 priority 110
standby 50 preempt

The same design is used for VLANs 10, 20, 30, 40, 50, 60 and 99.

# 10. OSPF Dynamic Routing

OSPF process 1 is used for dynamic routing over the routed /30 links. DSW1 uses router ID 3.3.3.3. OSPF is passive by default except on the routed interfaces connected to the campus routers.

router ospf 1
router-id 3.3.3.3
passive-interface default
no passive-interface FastEthernet0/4
no passive-interface FastEthernet0/5
network 10.255.1.0 0.0.0.3 area 0
network 10.255.2.4 0.0.0.3 area 0

# 11. DHCP Services

DSW1 provides DHCP pools for VLANs 10, 20, 30, 40, 50 and 60. Gateway and infrastructure addresses from .1 through .20 are excluded.

ip dhcp excluded-address 10.10.50.1 10.10.50.20
ip dhcp pool VLAN50-TRAINING
network 10.10.50.0 255.255.255.0
default-router 10.10.50.1
dns-server 8.8.8.8

Equivalent DHCP pools were configured for ADMIN, HR, FINANCE, DEVELOPMENT and SERVER VLANs.

# 12. EtherChannel and Trunking

DSW1 and DSW2 use LACP to create Port-channel1 from GigabitEthernet0/1 and GigabitEthernet0/2. The EtherChannel carries VLANs 10, 20, 30, 40, 50, 60 and 99.

interface Port-channel1
description LACP-TRUNK-TO-DSW2
switchport trunk allowed vlan 10,20,30,40,50,60,99
 
interface GigabitEthernet0/1
channel-group 1 mode active
interface GigabitEthernet0/2
channel-group 1 mode active

# 13. SSH Management

SSH version 2 is enabled on all routers and switches. Local authentication uses the administrative user account. VTY lines are configured for local login and SSH transport.

username admin privilege 15 secret <configured-secret>
ip ssh version 2
ip domain-name campus.local
 
line vty 0 4
login local
transport input ssh

exit

ASW1, ASW2 and ASW3 were additionally corrected so VTY lines 5–15 use local authentication and SSH transport. Plaintext passwords are intentionally not reproduced in this report.

# 14. Management PC

The Management PC is configured with a static IP because it is used for administrative access and should not depend on DHCP.

| Parameter | Value |
| --- | --- |
| IP Address | 10.10.99.10 |
| Subnet Mask | 255.255.255.0 |
| Default Gateway | 10.10.99.1 |
| DNS Server | 8.8.8.8 |
| VLAN | 99 – MANAGEMENT |
| Addressing | Static |

This address is within the Management VLAN and is permitted by the SSH management ACL.

# 15. Access Control and Security

## 15.1 TRAINING-to-SERVER ACL

ip access-list extended ACL_RESTRICT_TRAINING
deny ip 10.10.50.0 0.0.0.255 10.10.60.0 0.0.0.255
permit ip any any

The ACL blocks IP traffic from the TRAINING subnet to the SERVER subnet while allowing other IP traffic. ACL counters were observed increasing during testing.

## 15.2 SSH Management ACL

ip access-list standard ACL_SSH_MANAGEMENT
permit 192.168.99.0 0.0.0.255
 
line vty 0 4
access-class ACL_SSH_MANAGEMENT in
login local
transport input ssh

The SSH management ACL permits remote administration only from the Management VLAN. DSW2 was verified with this ACL applied to the VTY lines.

# 16. Verification and Testing

| Area | Verification Command | Expected / Observed Result |
| --- | --- | --- |
| Interfaces | show ip interface brief | Required interfaces and SVIs up/up |
| VLANs | show vlan brief | Required VLANs present |
| Trunks | show interfaces trunk | Required VLANs allowed |
| EtherChannel | show etherchannel summary | Port-channel1 operational with LACP members |
| HSRP | show standby brief | Virtual gateways and active/standby roles |
| OSPF | show ip ospf neighbor | OSPF adjacency established |
| Routing | show ip route | Connected and dynamic routes present |
| SSH | show ip ssh | SSH version 2.0 enabled |
| VTY | show running-config ¦ section line vty | Local login and SSH transport |
| ACL | show access-lists | ACL entries and hit counters |
| ACL attachment | show running-config ¦ include access-group | Used to verify interface/VTY attachment |
| Configuration save | write memory | Configuration saved |

# 17. SSH Testing

ssh -l admin 192.168.99.2
ssh -l admin 192.168.99.3
ssh -l admin 192.168.99.11
ssh -l admin 192.168.99.12
ssh -l admin 192.168.99.13

SSH testing from the Management PC was completed against DSW1, DSW2, ASW1, ASW2 and ASW3 using the static source address 192.168.99.10.

# 18. Troubleshooting Record

## 18.1 ACL Existed but Was Initially Not Shown as Applied to VLAN 50

DSW1 initially displayed ACL_RESTRICT_TRAINING under show access-lists, but show ip interface vlan 50 reported that no inbound or outgoing access list was set. show running-config | include access-group also returned no attachment. The full running configuration confirmed the ACL definition existed separately from the interface configuration.

interface vlan 50
ip access-group ACL_RESTRICT_TRAINING in
end
write memory

Packet Tracer continued to show inconsistent interface-level display during one verification attempt. Therefore, ACL hit counters were used as additional functional evidence. The ACL deny and permit entries accumulated matches. The same ACL policy was also configured on DSW2. This issue is documented as a project troubleshooting item rather than omitted.

## 18.2 EtherChannel Was Working but Port-Channel Was Initially Not Shown as Trunk

DSW1 initially showed that the LACP EtherChannel was operational, but show interfaces trunk did not display the Port-Channel as a trunk. The EtherChannel members Gi0/1 and Gi0/2 were successfully bundled into Po1.

The issue occurred because the trunk encapsulation was set to Auto. When switchport mode trunk was configured, Packet Tracer rejected the command because the encapsulation was not explicitly set.

interface port-channel 1

switchport trunk encapsulation dot1q

switchport mode trunk

switchport trunk allowed vlan 10,20,30,40,50,60,99

no shutdown

After configuring 802.1Q encapsulation and trunk mode, Po1 operated correctly as a trunk and carried VLANs 10, 20, 30, 40, 50, 60, and 99. The same issue was encountered and resolved on DSW2.

19. Final Network Status

- VLAN segmentation completed.

- HSRP gateway redundancy configured.

- OSPF dynamic routing configured.

- DHCP services configured for required VLANs.

- LACP EtherChannel configured between DSW1 and DSW2.

- SSH version 2 enabled across all network devices.

- Management PC configured with static address 192.168.99.10.

- SSH management ACL configured for VLAN 99.

- TRAINING-to-SERVER traffic restriction configured.

- Connectivity, protocol and security verification completed.

- Configurations saved using write memory.

# 20. Conclusion

The Redundant Multi-Floor Enterprise Campus Network was successfully implemented as an enterprise-style Cisco Packet Tracer solution. The completed design combines VLAN segmentation, HSRP redundancy, OSPF routing, LACP EtherChannel, DHCP, secure SSH management and ACL-based traffic controls. The Management VLAN provides a dedicated administration network, while the static Management PC provides a predictable source for secure SSH access. The documented troubleshooting process demonstrates practical verification and correction of configuration issues.

# Appendix A — Management Addressing

| Device | Management IP |
| --- | --- |
| Management PC | 192.168.99.10 |
| HSRP Virtual Gateway | 192.168.99.1 |
| DSW1 | 192.168.99.2 |
| DSW2 | 192.168.99.3 |
| ASW1 | 192.168.99.11 |
| ASW2 | 192.168.99.12 |
| ASW3 | 192.168.99.13 |

# Appendix B — Credentials

The local administrative username is admin on the project devices. Passwords Admin@123.

# Appendix C — Important Commands

show ip interface brief
show vlan brief
show interfaces trunk
show etherchannel summary
show standby brief
show ip ospf neighbor
show ip route
show ip ssh
show running-config | include username
show running-config | section line vty
show access-lists
show running-config | include access-group
show ip interface vlan 50
write memory

# Appendix D — Project  Log

| Stage | Implementation | Status |
| --- | --- | --- |
| Foundation | Topology and device roles | Complete |
| DSW | VLANs, trunks, Rapid-PVST, LACP | Complete |
| Routing | SVIs, HSRP and OSPF | Complete |
| Services | DHCP | Complete |
| Management | SSH v2 and local authentication | Complete |
| Security | SSH management ACL | Complete |
| Security | TRAINING-to-SERVER ACL | Complete |
| Management | Static Management PC 192.168.99.10 | Complete |
| Testing | Connectivity and protocol verification | Complete |
| Troubleshooting | ACL application/display issue and VTY correction | Documented |
| Documentation | Final project report and flow chart | Complete |