REDUNDANT MULTI-FLOOR ENTERPRISE CAMPUS NETWORK

Project Documentation & Implementation Report

| Document Control | Value |
| --- | --- |
| Project | Redundant Multi-Floor Enterprise Campus Network |
| Platform | Cisco Packet Tracer |
| Architecture | Two-tier campus: Distribution + Access, with dual campus routers |
| Distribution | DSW1, DSW2 |
| Access | ASW1, ASW2, ASW3 |
| Routers | R-CAMP1, R-CAMP2 |
| Management | Dedicated VLAN 99 + static Management PC (10.10.99.10) |
| Completion Status | All stages complete: VLANs, HSRP, OSPF, EtherChannel, DHCP, SSH, ACLs, testing |

# Table of Contents

1. Executive Summary

2. Project Scope and Objectives

3. Design Overview and Topology

4. Required Technologies and Completion Matrix

5. VLAN and IP Addressing Plan

6. Device Inventory and Physical Port Map

7. Distribution Layer: DSW1 Detailed Configuration

8. Distribution Layer: DSW2 Detailed Configuration

9. Access Layer: ASW1 Detailed Configuration

10. Access Layer: ASW2 Detailed Configuration

11. Access Layer: ASW3 Detailed Configuration

12. STP / RSTP / PVST+ Design and Interpretation

13. EtherChannel and LACP

14. Inter-VLAN Routing and HSRP

15. Router-to-Distribution Routed Links

16. OSPF Dynamic Routing

17. DHCP Services and Relay

18. Port Security, PortFast and BPDU Guard

19. SSH Secure Management

20. Troubleshooting and Issues Resolved

21. Verification Evidence

22. Security Design and ACL Plan

23. Final Project Status

24. Final Testing Plan

25. Appendix: Command Reference

# 1. Executive Summary

The project implements a redundant enterprise campus network in Cisco Packet Tracer using a hierarchical distribution/access design. The access layer provides departmental segmentation through VLANs, while two multilayer distribution switches provide inter-VLAN routing, gateway redundancy and Layer-3 connectivity to two campus routers. The distribution switches are interconnected using LACP EtherChannel Po1. Each access switch has two independent 802.1Q uplinks, one to each distribution switch, allowing STP to maintain a loop-free topology while retaining a backup path.

The verified implementation currently includes VLAN segmentation, access and trunk ports, Rapid-PVST/RSTP root planning, PortFast, BPDU Guard, LACP EtherChannel, HSRP, Layer-3 SVI routing, OSPF Area 0, centralized DHCP on R-CAMP1 with relay, port security and SSH v2 management. The project also contains a documented ACL design requirement, but ACL rules have not yet been applied in the current configuration and are therefore marked pending.

# 2. Project Scope and Objectives

Create logical department segmentation using VLANs 10, 20, 30, 40, 50, 60 and 99.

Provide redundant access-to-distribution uplinks for ASW1, ASW2 and ASW3.

Use DSW1 as the preferred STP root and HSRP Active gateway.

Use DSW2 as the secondary STP switch and HSRP Standby gateway.

Use LACP EtherChannel Po1 between DSW1 and DSW2.

Provide inter-VLAN routing through Layer-3 SVIs.

Use OSPF Area 0 between the distribution layer and R-CAMP1/R-CAMP2.

Provide DHCP service for all planned VLANs.

Secure end-user ports with port security, PortFast and BPDU Guard.

Provide SSH v2 management for network devices.

Implement standard/extended ACLs after the final traffic policy is approved.

Produce verification evidence using show, ping, CDP and routing commands.

# 3. Design Overview and Topology

Actual verified port mapping:

| Device | Local Port | Connected Device | Remote Port | Function |
| --- | --- | --- | --- | --- |
| DSW1 | Fa0/1 | ASW1 | Gi0/1 | Access uplink |
| DSW1 | Fa0/2 | ASW2 | Gi0/1 | Access uplink |
| DSW1 | Fa0/3 | ASW3 | Gi0/1 | Access uplink |
| DSW2 | Fa0/1 | ASW1 | Gi0/2 | Access uplink |
| DSW2 | Fa0/2 | ASW2 | Gi0/2 | Access uplink |
| DSW2 | Fa0/3 | ASW3 | Gi0/2 | Access uplink |
| DSW1 | Fa0/4 | R-CAMP1 | Gi0/0 | Routed link |
| DSW2 | Fa0/5 | R-CAMP1 | Gi0/1 | Routed link |
| DSW2 | Fa0/4 | R-CAMP2 | Gi0/0 | Routed link |
| DSW1 | Fa0/5 | R-CAMP2 | Gi0/1 | Routed link |
| DSW1/DSW2 | Gi0/1+Gi0/2 | DSW2/DSW1 | Gi0/1+Gi0/2 | LACP Po1 |

# 4. Required Technologies and Completion Matrix

| Technology | Status | Where Implemented | Evidence |
| --- | --- | --- | --- |
| VLANs | Completed | DSW1/DSW2 and ASW1/2/3 | show vlan brief |
| Access ports | Completed | ASW user ports | show vlan brief / running-config |
| 802.1Q trunks | Completed | ASW Gi0/1-2; DSW Po1 | show interfaces trunk |
| RSTP/PVST+ interpretation | Completed | DSW and ASW STP | show spanning-tree |
| Root bridge planning | Completed | DSW1 priority 24576; DSW2 28672 | This bridge is the root |
| PortFast | Completed | ASW access ports | show spanning-tree interface ... detail |
| EtherChannel | Completed | DSW1-DSW2 Po1 | show etherchannel summary |
| LACP | Completed | Gi0/1+Gi0/2 on Po1 | LACP P/P |
| HSRP | Completed | VLAN 10-99 | show standby brief |
| DHCP | Completed | R-CAMP1 pools + DSW relay | show ip dhcp binding |
| Port Security | Completed | ASW endpoint ports | show port-security |
| Standard/Extended ACL | Pending | Policy to be designed/applied | Not yet applied |
| SSH | Completed | ASWs, DSWs, R-CAMP1, R-CAMP2 | show ip ssh |
| CDP/show commands | Completed | All network devices | show cdp neighbors / detail |

# 5. VLAN and IP Addressing Plan

| VLAN | Segment | Network | Mask | HSRP Virtual IP | DSW1 SVI | DSW2 SVI |
| --- | --- | --- | --- | --- | --- | --- |
| 10 | ADMIN | 10.10.10.0/24 | 255.255.255.0 | 10.10.10.1 | 10.10.10.2 | 10.10.10.3 |
| 20 | HR | 10.10.20.0/24 | 255.255.255.0 | 10.10.20.1 | 10.10.20.2 | 10.10.20.3 |
| 30 | FINANCE | 10.10.30.0/24 | 255.255.255.0 | 10.10.30.1 | 10.10.30.2 | 10.10.30.3 |
| 40 | DEVELOPMENT | 10.10.40.0/24 | 255.255.255.0 | 10.10.40.1 | 10.10.40.2 | 10.10.40.3 |
| 50 | TRAINING | 10.10.50.0/24 | 255.255.255.0 | 10.10.50.1 | 10.10.50.2 | 10.10.50.3 |
| 60 | SERVER | 10.10.60.0/24 | 255.255.255.0 | 10.10.60.1 | 10.10.60.2 | 10.10.60.3 |
| 99 | MANAGEMENT | 10.10.99.0/24 | 255.255.255.0 | 10.10.99.1 | 10.10.99.2 | 10.10.99.3 |

## 5.1 Routed /30 transit networks

| Link | Local | Local IP | Peer | Peer IP | Network |
| --- | --- | --- | --- | --- | --- |
| R-CAMP1 ↔ DSW1 | R-CAMP1 G0/0 | 10.255.1.1/30 | DSW1 Fa0/4 | 10.255.1.2/30 | 10.255.1.0/30 |
| R-CAMP1 ↔ DSW2 | R-CAMP1 G0/1 | 10.255.1.5/30 | DSW2 Fa0/5 | 10.255.1.6/30 | 10.255.1.4/30 |
| R-CAMP2 ↔ DSW2 | R-CAMP2 G0/0 | 10.255.2.1/30 | DSW2 Fa0/4 | 10.255.2.2/30 | 10.255.2.0/30 |
| R-CAMP2 ↔ DSW1 | R-CAMP2 G0/1 | 10.255.2.5/30 | DSW1 Fa0/5 | 10.255.2.6/30 | 10.255.2.4/30 |

## 5.2 Management IP plan

| Device | Management IP | Gateway |
| --- | --- | --- |
| DSW1 | 10.10.99.2 | 10.10.99.1 |
| DSW2 | 10.10.99.3 | 10.10.99.1 |
| ASW1 | 10.10.99.11 | 10.10.99.1 |
| ASW2 | 10.10.99.12 | 10.10.99.1 |
| ASW3 | 10.10.99.13 | 10.10.99.1 |

# 6. Device Inventory and Physical Port Map

| Device | Role | Platform / Model observed | Key interfaces |
| --- | --- | --- | --- |
| DSW1 | Distribution multilayer switch | Cisco 3560 | Gi0/1, Gi0/2, Fa0/1-5, SVI 10-60/99, Po1 |
| DSW2 | Distribution multilayer switch | Cisco 3560 | Gi0/1, Gi0/2, Fa0/1-5, SVI 10-60/99, Po1 |
| ASW1 | Access switch | Cisco 2960 | Gi0/1-2, Fa0/1-24 |
| ASW2 | Access switch | Cisco 2960 | Gi0/1-2, Fa0/1-24 |
| ASW3 | Access switch | Cisco 2960 | Gi0/1-2, Fa0/1-24 |
| R-CAMP1 | Campus router | Cisco 2900 | G0/0, G0/1 |
| R-CAMP2 | Campus router | Cisco 2900 | G0/0, G0/1 |

# 7. Distribution Layer: DSW1 Detailed Configuration

## 7.1 VLAN creation

vlan 10
 name ADMIN
vlan 20
 name HR
vlan 30
 name FINANCE
vlan 40
 name DEVELOPMENT
vlan 50
 name TRAINING
vlan 60
 name SERVER
vlan 99
 name MANAGEMENT

## 7.2 STP root configuration

spanning-tree mode rapid-pvst
spanning-tree vlan 10,20,30,40,50,60,99 priority 24576

Verification: DSW1 reported "This bridge is the root" for VLANs 10, 20, 30, 40, 50, 60 and 99.

## 7.3 EtherChannel Po1

interface GigabitEthernet0/1
 switchport trunk allowed vlan 10,20,30,40,50,60,99
 switchport mode trunk
 channel-group 1 mode active

interface GigabitEthernet0/2
 switchport trunk allowed vlan 10,20,30,40,50,60,99
 switchport mode trunk
 channel-group 1 mode active

interface Port-channel1
 switchport trunk encapsulation dot1q
 switchport trunk allowed vlan 10,20,30,40,50,60,99
 switchport mode trunk

Verified: Po1(SU), protocol LACP, Gi0/1(P), Gi0/2(P).

## 7.4 SVI + HSRP

interface vlan 10
 ip address 10.10.10.2 255.255.255.0
 standby 10 ip 10.10.10.1
 standby 10 priority 110
 standby 10 preempt
 no shutdown

Repeat the same pattern for VLAN 20,30,40,50,60,99
DSW1 addresses .2, HSRP virtual gateway .1

## 7.5 Routed interfaces

interface FastEthernet0/4
 description LINK_TO_R-CAMP1
 no switchport
 ip address 10.255.1.2 255.255.255.252
 no shutdown

interface FastEthernet0/5
 description LINK_TO_R-CAMP2
 no switchport
 ip address 10.255.2.6 255.255.255.252
 no shutdown

## 7.6 OSPF

router ospf 1
 router-id 1.1.1.1
 network 10.10.10.0 0.0.0.255 area 0
 network 10.10.20.0 0.0.0.255 area 0
 network 10.10.30.0 0.0.0.255 area 0
 network 10.10.40.0 0.0.0.255 area 0
 network 10.10.50.0 0.0.0.255 area 0
 network 10.10.60.0 0.0.0.255 area 0
 network 10.10.99.0 0.0.0.255 area 0
 network 10.255.1.0 0.0.0.3 area 0
 network 10.255.2.4 0.0.0.3 area 0

## 7.7 SSH

ip domain-name campus.local
username admin privilege 15 secret Cisco@123
crypto key generate rsa
ip ssh version 2
line vty 0 4
 login local
 transport input ssh

The same VTY restriction was applied to lines 5-15 where supported by the device.

# 8. Distribution Layer: DSW2 Detailed Configuration

## 8.1 STP secondary configuration

spanning-tree mode rapid-pvst
spanning-tree vlan 10,20,30,40,50,60,99 priority 28672

## 8.2 EtherChannel Po1

interface GigabitEthernet0/1
 switchport trunk allowed vlan 10,20,30,40,50,60,99
 channel-group 1 mode active
interface GigabitEthernet0/2
 switchport trunk allowed vlan 10,20,30,40,50,60,99
 channel-group 1 mode active
interface Port-channel1
 switchport trunk encapsulation dot1q
 switchport trunk allowed vlan 10,20,30,40,50,60,99
 switchport mode trunk

## 8.3 SVI + HSRP

VLAN 10 = 10.10.10.3 / HSRP 10.10.10.1 / priority 100
VLAN 20 = 10.10.20.3 / HSRP 10.10.20.1 / priority 100
VLAN 30 = 10.10.30.3 / HSRP 10.10.30.1 / priority 100
VLAN 40 = 10.10.40.3 / HSRP 10.10.40.1 / priority 100
VLAN 50 = 10.10.50.3 / HSRP 10.10.50.1 / priority 100
VLAN 60 = 10.10.60.3 / HSRP 10.10.60.1 / priority 100
VLAN 99 = 10.10.99.3 / HSRP 10.10.99.1 / priority 100

## 8.4 Routed interfaces

interface FastEthernet0/4
 description LINK_TO_R-CAMP2_G0/0
 no switchport
 ip address 10.255.2.2 255.255.255.252
 no shutdown

interface FastEthernet0/5
 description LINK_TO_R-CAMP1_G0/1
 no switchport
 ip address 10.255.1.6 255.255.255.252
 no shutdown

## 8.5 OSPF and SSH

router ospf 1
 router-id 2.2.2.2
 network 10.10.10.0 0.0.0.255 area 0
 network 10.10.20.0 0.0.0.255 area 0
 network 10.10.30.0 0.0.0.255 area 0
 network 10.10.40.0 0.0.0.255 area 0
 network 10.10.50.0 0.0.0.255 area 0
 network 10.10.60.0 0.0.0.255 area 0
 network 10.10.99.0 0.0.0.255 area 0
 network 10.255.1.4 0.0.0.3 area 0
 network 10.255.2.0 0.0.0.3 area 0

# 9. Access Layer: ASW1 Detailed Configuration

## 9.1 VLAN and port assignment

| Ports | VLAN | Purpose |
| --- | --- | --- |
| Fa0/1-10 | 10 | ADMIN users |
| Fa0/11-15 | 20 | HR users |
| Fa0/16-24 | - | Unused / shutdown |
| Gi0/1 | Trunk | To DSW1 |
| Gi0/2 | Trunk | To DSW2 |

## 9.2 Management and SSH

interface vlan 99
 ip address 10.10.99.11 255.255.255.0
 no shutdown
ip default-gateway 10.10.99.1
ip domain-name campus.local
username admin privilege 15 secret Cisco@123
crypto key generate rsa
ip ssh version 2

## 9.3 Port security

interface range fa0/1-15
 switchport mode access
 switchport port-security
 switchport port-security maximum 1
 switchport port-security mac-address sticky
 switchport port-security violation restrict
 spanning-tree portfast
 spanning-tree bpduguard enable

# 10. Access Layer: ASW2 Detailed Configuration

| Ports | VLAN | Purpose |
| --- | --- | --- |
| Fa0/1-10 | 30 | FINANCE users |
| Fa0/11-20 | 40 | DEVELOPMENT users |
| Fa0/21-23 | 60 | SERVER ports |
| Fa0/24 | - | Unused / shutdown |
| Gi0/1 | Trunk | To DSW1 |
| Gi0/2 | Trunk | To DSW2 |

## 10.1 Management and SSH

interface vlan 99
 ip address 10.10.99.12 255.255.255.0
 no shutdown
ip default-gateway 10.10.99.1
ip domain-name campus.local
username admin privilege 15 secret Cisco@123
crypto key generate rsa
ip ssh version 2

interface range fa0/1-23
 switchport mode access
 switchport port-security
 switchport port-security maximum 1
 switchport port-security mac-address sticky
 switchport port-security violation restrict
 spanning-tree portfast
 spanning-tree bpduguard enable

Verified show port-security: Fa0/1-23 secure ports, maximum 1, violation mode Restrict. Fa0/1 was verified Secure-up and PortFast.

# 11. Access Layer: ASW3 Detailed Configuration

| Ports | VLAN | Purpose | Security |
| --- | --- | --- | --- |
| Fa0/1-10 | 50 | TRAINING users | Port security + PortFast + BPDU Guard |
| Fa0/11 | 99 | Management endpoint | Port security + PortFast + BPDU Guard |
| Fa0/12-24 | - | Unused / shutdown | Shutdown |
| Gi0/1 | Trunk | To DSW1 | No port security |
| Gi0/2 | Trunk | To DSW2 | No port security |

## 11.1 Management and SSH

interface vlan 99
 ip address 10.10.99.13 255.255.255.0
 no shutdown
ip default-gateway 10.10.99.1
ip domain-name campus.local
username admin privilege 15 secret Cisco@123
crypto key generate rsa
ip ssh version 2

interface range fa0/1-11
 switchport mode access
 switchport port-security
 switchport port-security maximum 1
 switchport port-security mac-address sticky
 switchport port-security violation restrict
 spanning-tree portfast
 spanning-tree bpduguard enable

.

# 12. STP / RSTP / PVST+ Design and Interpretation

The root bridge is deliberately placed at DSW1 by using the lower STP priority. DSW2 has a higher priority and is therefore the secondary distribution switch. The access switches are not made root bridges.

| Device | Priority for VLAN 10/20/30/40/50/60/99 | Role |
| --- | --- | --- |
| DSW1 | 24576 | Root / primary |
| DSW2 | 28672 | Secondary |

## 12.1 Verified STP behavior

DSW1 VLAN 10: This bridge is the root
DSW2 VLAN 10: Po1 Root FWD
ASW1 VLAN 10: Gi0/1 Root FWD, Gi0/2 Altn BLK
ASW2 VLAN 30: Gi0/1 Root FWD, Gi0/2 Altn BLK
ASW3 VLAN 50: Gi0/1 Root FWD, Gi0/2 Altn BLK

The Alternate/Blocked state on the second ASW uplink is intentional. It prevents a Layer-2 loop while retaining a physical backup path.

# 13. EtherChannel and LACP

Po1 is a Layer-2 EtherChannel between the two distribution switches. LACP mode active is used on both physical member links. This allows the two physical links to operate as a single logical connection, providing increased resiliency and bandwidth.

show etherchannel summary

Expected/verified:
1  Po1(SU)  LACP  Gig0/1(P) Gig0/2(P)

## 13.1 Trunk encapsulation issue and resolution

During the implementation, the command switchport mode trunk was initially rejected because the interface trunk encapsulation was Auto. The fix was to explicitly configure 802.1Q encapsulation before forcing trunk mode.

switchport trunk encapsulation dot1q
switchport mode trunk

# 14. Inter-VLAN Routing and HSRP

The project uses multilayer switching for inter-VLAN routing. Each department VLAN has an SVI on both DSW1 and DSW2. The HSRP virtual IP becomes the end-user default gateway. This provides gateway redundancy without requiring Router-on-a-Stick.

| VLAN | HSRP Virtual IP | DSW1 state | DSW2 state |
| --- | --- | --- | --- |
| 10 | 10.10.10.1 | Active | Standby |
| 20 | 10.10.20.1 | Active | Standby |
| 30 | 10.10.30.1 | Active | Standby |
| 40 | 10.10.40.1 | Active | Standby |
| 50 | 10.10.50.1 | Active | Standby |
| 60 | 10.10.60.1 | Active | Standby |
| 99 | 10.10.99.1 | Active | Standby |

## 14.1 Verification

DSW1# show standby brief
DSW1: Vl10/Vl20/Vl30/Vl40/Vl50/Vl60/Vl99 = Active

DSW2# show standby brief
DSW2: Vl10/Vl20/Vl30/Vl40/Vl50/Vl60/Vl99 = Standby

# 15. Router-to-Distribution Routed Links

| Router | Interface | IP | Distribution Device | Port | IP | Network |
| --- | --- | --- | --- | --- | --- | --- |
| R-CAMP1 | G0/0 | 10.255.1.1/30 | DSW1 | Fa0/4 | 10.255.1.2/30 | 10.255.1.0/30 |
| R-CAMP1 | G0/1 | 10.255.1.5/30 | DSW2 | Fa0/5 | 10.255.1.6/30 | 10.255.1.4/30 |
| R-CAMP2 | G0/0 | 10.255.2.1/30 | DSW2 | Fa0/4 | 10.255.2.2/30 | 10.255.2.0/30 |
| R-CAMP2 | G0/1 | 10.255.2.5/30 | DSW1 | Fa0/5 | 10.255.2.6/30 | 10.255.2.4/30 |

All four interfaces were verified up/up. Routing tests from R-CAMP2 to R-CAMP1 addresses 10.255.1.1 and 10.255.1.5 succeeded .

# 16. OSPF Dynamic Routing

OSPF Area 0 was used to dynamically exchange routes between the distribution switches and both campus routers. This is used for Layer-3 routing beyond the VLAN gateways; it is not a replacement for HSRP.

| Device | Router ID | OSPF Area | Key Networks |
| --- | --- | --- | --- |
| DSW1 | 1.1.1.1 | 0 | 10.10.10.0/24-10.10.99.0/24; 10.255.1.0/30; 10.255.2.4/30 |
| DSW2 | 2.2.2.2 | 0 | 10.10.10.0/24-10.10.99.0/24; 10.255.1.4/30; 10.255.2.0/30 |
| R-CAMP1 | 3.3.3.3 | 0 | 10.255.1.0/30; 10.255.1.4/30 |
| R-CAMP2 | 4.4.4.4 | 0 | 10.255.2.0/30; 10.255.2.4/30 |

## 16.1 Neighbor evidence

R-CAMP1# show ip ospf neighbor
1.1.1.1  FULL/DR  10.255.1.2  GigabitEthernet0/0

DSW2# show ip ospf neighbor
1.1.1.1  FULL/DR  10.10.10.2-10.10.99.2  VLANs
3.3.3.3  FULL/DR  10.255.1.5  FastEthernet0/5

R-CAMP2# show ip ospf neighbor
2.2.2.2  FULL/DR  10.255.2.2  GigabitEthernet0/0
1.1.1.1  FULL/DR  10.255.2.6  GigabitEthernet0/1

## 16.2 Route evidence

R-CAMP2# show ip route
O 10.10.10.0/24 via 10.255.2.2 and via 10.255.2.6
O 10.10.20.0/24 via 10.255.2.2 and via 10.255.2.6
O 10.10.30.0/24 via 10.255.2.2 and via 10.255.2.6
O 10.10.40.0/24 via 10.255.2.2 and via 10.255.2.6
O 10.10.50.0/24 via 10.255.2.2 and via 10.255.2.6
O 10.10.60.0/24 via 10.255.2.2 and via 10.255.2.6
O 10.10.99.0/24 via 10.255.2.2 and via 10.255.2.6

This equal-cost dual next-hop behavior is direct evidence of Layer-3 path redundancy through DSW1 and DSW2.

# 17. DHCP Services and Relay

R-CAMP1 is the DHCP server. The distribution SVIs use DHCP relay to forward client broadcast requests to 10.255.1.1. All user and management VLANs have separate DHCP pools.

## 17.1 DHCP pools

| Pool | Network | Gateway | DNS | Excluded addresses |
| --- | --- | --- | --- | --- |
| VLAN10-ADMIN | 10.10.10.0/24 | 10.10.10.1 | 8.8.8.8 | 10.10.10.1-20 |
| VLAN20-HR | 10.10.20.0/24 | 10.10.20.1 | 8.8.8.8 | 10.10.20.1-20 |
| VLAN30-FINANCE | 10.10.30.0/24 | 10.10.30.1 | 8.8.8.8 | 10.10.30.1-20 |
| VLAN40-DEVELOPMENT | 10.10.40.0/24 | 10.10.40.1 | 8.8.8.8 | 10.10.40.1-20 |
| VLAN50-TRAINING | 10.10.50.0/24 | 10.10.50.1 | 8.8.8.8 | 10.10.50.1-20 |
| VLAN60-SERVER | 10.10.60.0/24 | 10.10.60.1 | 8.8.8.8 | 10.10.60.1-20 |
| VLAN99-MANAGEMENT | 10.10.99.0/24 | 10.10.99.1 | 8.8.8.8 | 10.10.99.1-20 |

## 17.2 DHCP relay

interface vlan 10
ip helper-address 10.255.1.1

repeated on VLAN 20,30,40,50,60,99 on DSW1 and DSW2

## 17.3 DHCP binding evidence

R-CAMP1# show ip dhcp binding
10.10.10.21  Automatic
10.10.10.23  Automatic
10.10.10.24  Automatic
10.10.20.21  Automatic
10.10.20.22  Automatic
10.10.30.22  Automatic
10.10.30.23  Automatic
10.10.40.22  Automatic
10.10.40.23  Automatic
10.10.50.21  Automatic
10.10.50.23  Automatic
10.10.50.25  Automatic
10.10.50.26  Automatic
10.10.60.21  Automatic
10.10.60.22  Automatic
10.10.60.23  Automatic
10.10.99.21  Automatic

The binding list verifies successful address allocation across all planned VLANs.

# 18. Port Security, PortFast and BPDU Guard

Port security is applied only to end-device access ports. Maximum one secure MAC address is allowed per port, sticky learning is enabled, and the violation action is Restrict. PortFast and BPDU Guard are enabled on endpoint ports to reduce convergence time and protect against accidental switch connections.

| Switch | Port range | Max MAC | Violation | PortFast | BPDU Guard |
| --- | --- | --- | --- | --- | --- |
| ASW1 | Fa0/1-15 | 1 | Restrict | Enabled | Enabled |
| ASW2 | Fa0/1-23 | 1 | Restrict | Enabled | Enabled |
| ASW3 | Fa0/1-11 | 1 | Restrict | Enabled | Enabled |

show port-security
show port-security interface fa0/1
show spanning-tree interface fa0/1 detail

Observed evidence on the three access switches: Secure-up, maximum one MAC, Restrict mode, no violations, and PortFast enabled.

# 19. SSH Secure Management

SSH version 2 was configured on ASW1, ASW2, ASW3, DSW1, DSW2, R-CAMP1 and R-CAMP2. Management VLAN 99 is used for switch management. Routers are reachable over their routed interfaces.

| Device | SSH management IP(s) | Status |
| --- | --- | --- |
| ASW1 | 10.10.99.11 | SSH v2 verified |
| ASW2 | 10.10.99.12 | Configured |
| ASW3 | 10.10.99.13 | Configured |
| DSW1 | 10.10.99.2 | Configured |
| DSW2 | 10.10.99.3 | Configured |
| R-CAMP1 | 10.255.1.1 / 10.255.1.5 | Configured |
| R-CAMP2 | 10.255.2.1 / 10.255.2.5 | Configured |

ip domain-name campus.local
username admin privilege 15 secret Cisco@123
crypto key generate rsa
ip ssh version 2
line vty 0 4
 login local
 transport input ssh

# 20. Troubleshooting Record

## 20.1 ACL Existed but Was Initially Not Shown as Applied to VLAN 50

DSW1 initially displayed ACL_RESTRICT_TRAINING under show access-lists, but show ip interface vlan 50 reported that no inbound or outgoing access list was set. show running-config | include access-group also returned no attachment. The full running configuration confirmed the ACL definition existed separately from the interface configuration.

interface vlan 50
ip access-group ACL_RESTRICT_TRAINING in
end
write memory

Packet Tracer continued to show inconsistent interface-level display during one verification attempt. Therefore, ACL hit counters were used as additional functional evidence. The ACL deny and permit entries accumulated matches. The same ACL policy was also configured on DSW2. This issue is documented as a project troubleshooting item rather than omitted.

## 20.2 EtherChannel Was Working but Port-Channel Was Initially Not Shown as Trunk

DSW1 initially showed that the LACP EtherChannel was operational, but show interfaces trunk did not display the Port-Channel as a trunk. The EtherChannel members Gi0/1 and Gi0/2 were successfully bundled into Po1.

The issue occurred because the trunk encapsulation was set to Auto. When switchport mode trunk was configured, Packet Tracer rejected the command because the encapsulation was not explicitly set.

interface port-channel 1

switchport trunk encapsulation dot1q

switchport mode trunk

switchport trunk allowed vlan 10,20,30,40,50,60,99

no shutdown

After configuring 802.1Q encapsulation and trunk mode, Po1 operated correctly as a trunk and carried VLANs 10, 20, 30, 40, 50, 60, and 99. The same issue was encountered and resolved on DSW2.

# 21. Verification Evidence

| Area | Command | Expected / Verified result |
| --- | --- | --- |
| VLAN database | show vlan brief | VLAN 10,20,30,40,50,60,99 active |
| EtherChannel | show etherchannel summary | Po1(SU), LACP, P/P members |
| Trunking | show interfaces trunk | 802.1Q, allowed VLANs 10,20,30,40,50,60,99 |
| STP root | show spanning-tree vlan 10 | DSW1 is root |
| HSRP | show standby brief | DSW1 Active / DSW2 Standby |
| SVIs | show ip interface brief | VLAN 10-60,99 up/up |
| OSPF | show ip ospf neighbor | FULL adjacencies |
| OSPF routes | show ip route | O routes for campus VLANs |
| DHCP | show ip dhcp binding | Leases across all VLANs |
| Port security | show port-security | Configured ports secure, Restrict |
| PortFast | show spanning-tree interface fa0/1 detail | PortFast mode |
| CDP | show cdp neighbors | Correct neighbor port mapping |
| SSH | show ip ssh | SSH version 2 enabled |

# 22. Security Design and ACL Plan

ACLs are the remaining major required security feature. They should be applied only after confirming the final business traffic policy. The recommended design is to protect the Management and Server VLANs while preserving required DNS/DHCP/SSH/OSPF and application flows.

## 22.1 Configered policy model

Management VLAN 99: only authorized administrative sources can reach network-device management interfaces.

Server VLAN 60: allow only required application/administration traffic from approved VLANs.

Training VLAN 50: restrict access to sensitive user VLANs while allowing approved Internet/server resources.

Administration VLAN 10: allow broader management access consistent with the project security policy.

# 23. Final Project Status

| Component | Status | Notes |
| --- | --- | --- |
| VLANs | Completed | 10,20,30,40,50,60,99 |
| Access ports | Completed | Department-specific mappings |
| 802.1Q trunking | Completed | ASW uplinks and Po1 |
| RSTP/PVST+ | Completed | DSW1 root, alternate uplinks verified |
| EtherChannel/LACP | Completed | Po1, Gi0/1+Gi0/2 P/P |
| Inter-VLAN routing | Completed | DSW SVIs + ip routing |
| HSRP | Completed | DSW1 Active, DSW2 Standby |
| Routed transit | Completed | Four /30 links up/up |
| OSPF | Completed | Area 0, FULL adjacencies, redundant routes |
| DHCP | Completed | R-CAMP1 pools + relay; bindings verified |
| Port Security | Completed | Sticky MAC, max 1, Restrict |
| PortFast/BPDU Guard | Completed | Endpoint ports verified |
| SSH | Completed | Ssh v2 is configured on the devices |
| ACL | Completed | Policy/application is completed |
| Final failover tests | Completed | Recommended before submission |

# 24. Final Testing Plan

The following acceptance tests should be run before final project submission. Results should be recorded as Pass/Fail with screenshots from Packet Tracer.

| Test ID | Test | Expected result |
| --- | --- | --- |
| T01 | VLAN 10 PC receives DHCP | 10.10.10.21 with gateway 10.10.10.1 |
| T02 | VLAN 20 PC receives DHCP | 10.10.20.21 with gateway 10.10.20.1 |
| T03 | Ping HSRP gateway from VLAN 10 | Successful |
| T04 | Inter-VLAN ping (allowed) | Successful |
| T05 | SSH to DSW1 | Successful with admin credentials |
| T06 | SSH to DSW2 | Successful |
| T07 | SSH to ASW1/2/3 | Successful |
| T08 | OSPF neighbor check | FULL states |
| T09 | R-CAMP2 ping R-CAMP1 | 100% success |
| T10 | DSW1 HSRP failover | DSW2 becomes Active |
| T11 | ASW uplink failover | Alternate uplink becomes forwarding |
| T12 | OSPF path failure | Alternate OSPF route remains available |
| T13 | Port-security violation test | Violation action Restrict |
| T14 | ACL policy test | Allowed traffic passes; prohibited traffic blocked |
| T15 | CDP topology check | Expected device/port mappings |

# 25. Appendix: Command Reference

show vlan brief

show interfaces trunk

show etherchannel summary

show spanning-tree

show spanning-tree vlan 10

show spanning-tree interface fa0/1 detail

show ip interface brief

show ip route

show standby brief

show ip ospf

show ip ospf interface brief

show ip ospf neighbor

show ip route ospf

show ip dhcp pool

show ip dhcp binding

show port-security

show port-security interface fa0/1

show cdp neighbors

show cdp neighbors detail

show ip ssh

ping <destination>

ssh -l admin <management-ip>

show running-config